4.X SDK

Development Manual

Issue: V1.0.0

Date: 2019-03-25

Copyright © Hangzhou Tuya Inc. 2019 All rights reserved.

No part of this document may be reproduced or transmitted in any form or by any means without prior written consent of Hangzhou Tuya Inc.

Trademark Statement

, Tuya, and other Tuya trademarks all belong to Hangzhou Tuya Inc.

All other trademarks or registered trademarks mentioned in this document belong to their respective owners.

Notice

The purchased products, services, and features are stipulated in the contract made between Tuya and the customer. All or part of the products, services, and features described in this document may not be within the purchase scope or the usage scope. Unless otherwise agreed in the contract, all statements, information, and recommendations in this document are provided "AS IS" without warranties, guarantees, or representations of any kind, either express or implied.

The information in this document is subject to change without notice. Every effort has been made in the preparation of this document to ensure accuracy of the content, but all statements, information, and recommendations in this document do not constitute a warranty of any kind, express or implied.

Hangzhou Tuya Inc.

Address: 		Room 701, Building 3, Zheshang Wealth Center, No. 87 Gudun Road, Xihu District, Hangzhou

Website:		https://www.tuya.com

Tel: 			400-881-8611

Email:			service@tuya.com

About This Document

Overview

The Tuya 4.X SDK is solution code that the Tuya IPC Dept has developed and provided for interconnection with the Tuya cloud platform. It provides services such as device network configuration, app interaction, Internet Protocol camera (IPC) local storage, cloud storage, Echo Show service, Chromecast service, and point to point (P2P) transmission.

SDK Version

The product version used in this document is as follows.

| SDK Name | SDK Version |
| --- | --- |
| IPC Device SDK | 4.X SDK |

Intended Audience

This document is primarily intended for:

- Technical support engineers

- IPC software development engineers

Symbol Conventions

The symbols found in this document are defined as follows.

| Symbol | Description |
| --- | --- |
| - NOTICE | Indicates a potentially hazardous situation which, if not avoided, could result in device performance deterioration or unanticipated results during SDK startup. |
| ADVICE | Indicates a suggestion that can help you solve a problem or save your time. |
| NOTE | Calls attention to important information, best practices, and tips. |

Change History

Changes between document issues are cumulative. The latest document issue contains all the changes made in earlier issues.

Document Information

| Issue | Description | Prepared By | Date | Remarks |
| --- | --- | --- | --- | --- |
| V1.0.0 | This is the first revision. | Song Ping | 2019-03-25 | None |

Contents

Overview	3

SDK Version	3

Intended Audience	3

Symbol Conventions	3

Change History	3

Document Information	3

Contents	4

1 SDK Overview	5

1.1 Overview	5

1.2 Basic Terms	5

1.3 Network Configuration Modes	6

2 Demo Operation	8

2.1 Building a Demo Environment	8

2.2 Modifying the Demo Code	8

2.3 Running the Demo Code	8

3 SDK Network Configuration Development	10

3.1 Overview	10

3.2 Basic Parameters	10

3.3 Wi-Fi Access	11

3.4 QR Code Net-pairing	13

3.4.1 Development Process	13

3.5 AP Mode	15

3.6 Wired Network Configuration	17

4 Audio and Video Development	18

4.1 Real-time Preview Development	18

4.2 Local Video Development	19

5 DP Development	21

5.1 Overview	21

5.2 OSD Watermark DP Development	22

5.3 PIR DP Development	22

5.4 Motion Detection Development	22

6 Doorbell DP Development	24

6.1 Low Power-consuming Sleep DP Development	24

7 OTA DP Development	26

7.1 OTA Development Process	26

8 Cloud Storage Capability Development	28

8.1 Development Process	28

9 Echo Show and Chromecast DP Development	30

9.1 Echo Show DP Development Process	30

9.2 Chromecast DP Development Process	30

10 Head Shake Camera Movement Development	32

10.1 Head Shake Camera DP Description	32

10.2 Interface Description	32

10.3 DP Configuration Protocol Examples	33

1 SDK Overview

1.1 Overview

- Figure 1-1 shows the software development kit (SDK) service process. After obtaining an SDK, use an SDK module to register the SDK service. After establishing a service channel, send a device's audio and video information in required formats to the cache in the SDK. You can then operate the device through an app. Based on the original audio and video data, you can use the SDK and app to enable device data storage, point-to-point transmission, and third-party access, such as access to Echo Show and Chromecast.

- Figure 1-1 SDK service process

1.2 Basic Terms

- The following table describes terms used in this manual.

| Term | Description |
| --- | --- |
| Network configuration | Network configuration for a device establishes a communication link between the device, app, and Tuya Cloud. |
| PID | A product ID (PID) identifies a product type and associates with the product functions. IPC devices of the same type have the same PID. |
| UUID | A universally unique identifier (UUID) uniquely identifies a machine. |
| Authkey | A device's authkey is the service code indicating that the device has registered with Tuya Cloud and can use Tuya Cloud services. |
| P2P ID | The cloud automatically allocates the point-to-point service ID (P2P ID) of an SDK 3.0.0 or later. |
| Token | A token is an identification code generated by the cloud server during the QR code net-pairing process. It is valid for 10 minutes. |
| DP | A data point (DP) represents a function for data exchange between a device and the server. |
| Chromecast | Google Chromecast is a media streaming device that works with the Google Home Hub for real-time video transmission. |
| Echo Show | Echo Show is a smart speaker that Amazon has developed to support real-time video transmission. |
| IFTTT | If This Then That (IFTTT) is a free web-based service that allows you to connect to devices of other brands, for example, BMW. |
| OTA | An over-the-air (OTA) upgrade is an online device upgrade method. |

1.3 Network Configuration Modes

- Wi-Fi access

Connect your mobile phone to a 2.4 GHz Wi-Fi network. Fill in the Wi-Fi password and tap Next. The router will send a broadcast packet over the Wi-Fi network.

The camera receives the packet and sends it to the SDK, which then verifies and parses the packet into network access information to instruct the camera to connect to the network. The network access information contains the Wi-Fi account, Wi-Fi password, and token.

- NOTE: For details about the development process, see section 3.3 "Wi-Fi Access."

Figure 1-2 Wi-Fi access process

- QR code net-pairing

The camera scans and recognizes a QR code in the app and obtains the network access information. The network access information contains the Wi-Fi account, Wi-Fi password, and token.

- NOTE: For details about the development process, see section 3.4 "QR Code Net-pairing." The distance between the mobile phone and the camera is 20 cm to 30 cm.

Figure 1-3 QR code net-pairing process

- AP mode

Connect your mobile phone to a 2.4 GHz Wi-Fi network, fill in the Wi-Fi password, and tap Next. Connect the mobile phone to the camera's hotspot named "smart life-XXX" and send network access information to the camera.

After receiving the information, the camera will connect to the network. The network access information contains the Wi-Fi account, Wi-Fi password, and token.

- NOTE: For details about the development process, see section 3.5 "AP Mode."

Figure 1-4 AP mode process

- Wired network configuration

Use a network cable to connect the camera to a 2.4 GHz or 5 GHz Wi-Fi router. Connect your mobile phone to the Wi-Fi, and tap Add Device.

Tap Search Device and wait for the camera to access the network.

- NOTE: For details about the development process, see section 3.6 "Wired Network Configuration."

Figure 1-5 Wired network configuration process

2 Demo Operation

2.1 Building a Demo Environment

- Build a Ubuntu development environment. Ensure that the GNU compiler collection (GUU) has been installed. Run the following commands to check whether the environment meets the requirements.

$ apt-get install lib32ncurses5 lib32z1

$ apt-get install lib32stdc++6

- Log in to GitHub at https://github.com/TuyaInc/TUYA_IPC_SDK to download the Ubuntu demo SDK, as shown in the following figure.

- NOTE: The Ubuntu demo SDK supports a 64-bit operating system only.

2.2 Modifying the Demo Code

- Change the following macro definitions in the user_main.c file to the UUID and authkey for which you have applied.

- NOTE: Contact Tuya business personnel to apply for a UUID and authkey.

#define IPC_APP_UUID            "tuyaOneUuidForOneDevice"

#define IPC_APP_AUTHKEY         "tuyaOneAuthkeyForOneUUID"

- Change the following macro definition in the tuya_ipc_wifi_demo.c file to the name of the actual device NIC.

#define WLAN_DEV    "wlan0"

2.3 Running the Demo Code

- Download the libz library at https://superb-sea2.dl.sourceforge.net/project/libpng/zlib/1.2.11/zlib-1.2.11.tar.gz. Compile it into a .a library file, and store the .a library file to the libs folder.

- Run the following command to compile and generate an executable file.

- NOTE: The executable file is stored in the output folder that is generated after the compilation.

$ ./build_app.sh demo_tuya_ipc wifi

- Decompress the rawfiles file in the SDK's docs folder and copy the file to the output folder.

- Install the Tuya Smart app on your mobile phone, search for the camera in the app, and generate a QR code. Use WeChat to scan the QR code

to obtain the network configuration information.

- NOTE: AYm1YVV5jupJcF is a token, which is valid for 10 minutes.

{"s":"Tuya-Test","p":"88888888","t":"AYm1YVV5jupJcF"}

- Open the output folder, and run the following command in the directory where the tuya_ipc_demo_tuya_ipc file is stored.

- NOTE: Replace xxxx in the following command with the complete token.

$ ./ tuya_ipc_demo_tuya_ipc -m 2 -r “../” -t “xxxx”

- On the QR code page of the app, tap Next. Wait until the demo network configuration has completed. The following figure shows a successful demo network configuration page.

3 SDK Network Configuration Development

3.1 Overview

Tuya SDK has built a complete framework of functions for network configuration. This chapter describes the network configuration process and use of each framework function. You are advised to use corresponding framework functions to develop DPs.

- NOTE: The SDK library file contains the calling logic for each framework function. You do not need to develop it.

3.2 Basic Parameters

- Modify the save paths of the DB, OTA, and local video files. The macro definitions are as follows:

#define IPC_APP_STORAGE_PATH    "/tmp/"

#define IPC_APP_UPGRADE_FILE    "/tmp/upgrade.file"

#define IPC_APP_SD_BASE_PATH    "/tmp/"

- NOTE: The device's network configuration information is stored in the DB file. Save the DB file to a path where data will not be lost in case of a power failure.

Check whether there is a slash (/) at the end of the save paths of the DB file and local video file.

- Modify the macro definitions of PID, UUID, and authkey.

#define IPC_APP_PID             "gpkguYNp7yI4k413"

#define IPC_APP_UUID            "tuyaOneUuidForOneDevice"

#define IPC_APP_AUTHKEY         "tuyaOneAuthkeyForOneUUID"

- NOTE: Contact a project manager to obtain the PID.

Contact Tuya business personnel to obtain the UUID and authkey.

- Modify the macro definition of the version number.

#define IPC_APP_VERSION         "1.2.3"

- NOTE: The version number is in the xx.xx.xx format.

3.3 Wi-Fi Access

3.3.1 Wi-Fi Access Process Diagram

- NOTE: Complete the Wi-Fi access process step by step.

3.3.2 Development Process

- In the IPC_APP_Init_SDK function, select the WIFI_INIT_AUTO mode.

- The SDK calls the hwl_wf_wk_mode_get function, as shown in the following figure.

- NOTE: Configure Wi-Fi module status querying in the function based on the actual situation.

- The SDK calls the hwl_wf_all_ap_scan function and specifies the Wi-Fi module channel scanning result in the AP_IF_S structure, as shown in the following figure.

- NOTE: Configure Wi-Fi module channel scanning in the function based on the actual situation.

- The SDK calls the hwl_wf_sniffer_set function and specifies the information about the air interface packet received by the Wi-Fi module, as shown in the following figure.

- NOTE: Configure a Wi-Fi module sniffer status query method in the function based on the actual situation.

- After verifying the air interface packet data that the Wi-Fi module provides, the SDK calls the hwl_wf_wk_mode_set function to set the Wi-Fi status to the station mode.

- NOTE: Enable a Wi-Fi module mode in the function based on the actual situation.

- The SDK calls the hwl_wf_station_connect function to enable network access.

- NOTE: Enable Wi-Fi module network access in the function based on the actual situation.

- After the Wi-Fi module is connected to the network, call the hwl_wf_station_stat_get function to inform the SDK that an IP address has been obtained.

- NOTE: This interface is a high-frequency interface and the data needs to be returned in real time.

3.4 QR Code Net-pairing

3.4.1 Development Process

- In the IPC_APP_Init_SDK function, select the WIFI_INIT_AUTO mode.

- The SDK calls the hwl_wf_wk_mode_get function, as shown in the following figure.

- NOTE: Configure Wi-Fi module status querying in the function based on the actual situation.

- The SDK calls the hwl_wf_sniffer_set function to obtain the QR code scanning result, as shown in the following figure.

- NOTE: Call the zbar library in the __tuya_linux_get_snap_qrcode function to recognize the QR code based on the actual situation.

For the use of the ZBar library, see section 3.4.2 "ZBar Library Calling Example."

- After the SDK has obtained the QR code parsing result, call the hwl_wf_wk_mode_set function to set the Wi-Fi status to the station mode.

- NOTE: Enable a Wi-Fi module mode in the function based on the actual situation.

- The SDK calls the hwl_wf_station_connect function to enable network access.

- NOTE: Enable Wi-Fi module network access in the function based on the actual situation.

- After the Wi-Fi module is connected to the network, call the hwl_wf_station_stat_get function to inform the SDK that an IP address has been obtained.

- NOTE: This interface is a high-frequency interface and the data needs to be returned in real time.

3.4.2 ZBar Library Calling Example

ZBar is a barcode or QR code-scanning tool that supports camera and image scanning and multiple platforms. The tool provides an application programming interface (API) for QR code scanning.

Download the ZBar library at https://ayera.dl.sourceforge.net/project/zbar/zbar/0.10/zbar-0.10.tar.bz2.

Reference Code

int qrcode_parse_from_buffer(void *y8data, int w, int h, char* pqrcode)

{

int ret = -1;

zbar_image_scanner_t *scanner = NULL;

if(y8data == NULL || pqrcode == NULL)

{

return -1;

}

scanner = zbar_image_scanner_create();

uint32_t srcfmt = fourcc('Y','8','0','0');

zbar_image_t *image = zbar_image_create();

zbar_image_set_userdata(image, NULL);

zbar_image_set_format(image, srcfmt);

zbar_image_set_size(image, w, h);

zbar_image_set_data(image, y8data, w * h, zbar_image_get_userdata);

int n = zbar_scan_image(scanner, image);

const zbar_symbol_t *symbol = zbar_image_first_symbol(image);

if (NULL == symbol)

{

goto exit;

}

for(; symbol; symbol = zbar_symbol_next(symbol))

{

zbar_symbol_type_t typ = zbar_symbol_get_type(symbol);

const char *data = zbar_symbol_get_data(symbol);

if(data != NULL)

{

strncpy(pqrcode, data, zbar_symbol_get_data_length(symbol));

ret = 0;

printf("decoded %s symbol \"%s\"\n", zbar_get_symbol_name(typ), data);

}

}

exit:

zbar_image_destroy(image);

zbar_image_scanner_destroy(scanner);

return ret;

}

ADVICE: 	Scan the QR code as prompted by the app. Place your mobile phone at an appropriate distance from the camera.

A resolution of 320 x 240 pixels or higher is acceptable for QR code scanning.

During QR code scanning, disable irrelevant applications to save the device's resources.

3.5 AP Mode

- In the IPC_APP_Init_SDK function, select the WIFI_INIT_AP mode.

- The SDK calls the hwl_wf_wk_mode_get function, as shown in the following figure.

- NOTE: Configure Wi-Fi module status querying in the function based on the actual situation.

- The SDK calls the hwl_wf_ap_start function to start the AP mode, as shown in the following figure.

- NOTE: Enable the AP mode based on settings of the WF_AP_CFG_IF_S structure in the function.

- After receiving the AP mode information, the SDK calls the hwl_wf_wk_mode_set function to set the Wi-Fi status to the station mode.

- NOTE: Enable a Wi-Fi module mode in the function based on the actual situation.

- The SDK calls the hwl_wf_station_connect function to enable network access.

- NOTE: Enable Wi-Fi module network access in the function based on the actual situation.

- After the Wi-Fi module is connected to the network, call the hwl_wf_station_stat_get function to inform the SDK that an IP address has been obtained.

- NOTE: This interface is a high-frequency interface and the data needs to be returned in real time.

3.6 Wired Network Configuration

- Call the IPC_APP_Init_SDK (VOID) function to initialize the SDK.

- The SDK calls the hwl_bnw_station_conn function to obtain the connection status of the device's port.

- NOTE: Develop a port connection status query method in the function based on the actual situation.

- After obtaining the port connection status, the SDK calls the hwl_bnw_get_ip function to obtain the device's IP address.

- NOTE: Develop a device IP address query method in the function based on the actual situation.

- The SDK calls the hwl_bnw_get_mac function to obtain the MAC address of the device.

- NOTE: Develop a device MAC address query method in the function based on the actual situation.

- The SDK sends the obtained IP address and MAC address in broadcast mode through the router. Connect your mobile phone to the router's Wi-Fi.

The mobile phone can then receive the broadcast information to obtain a token from the server, which will be sent to the device for network access.

4 Audio and Video Development

4.1 Real-time Preview Development

When the SDK is initialized, a 10s audio and video ring buffer is created by default. Send the audio and video data that the device collects to ringbuffer. The SDK library file supports real-time data sending. You do not need to develop it.

Development Process

- Confirm the video parameter information in the IPC_APP_Set_Media_Info function.

- NOTE: In the following example, high definition (HD) video is enabled. Pay attention to the comments in a red font.

- Confirm the audio parameter information in the IPC_APP_Set_Media_Info function.

- NOTE: The following example uses default audio parameter settings. Pay attention to the comments in a red font.

The maximum length of an uploaded audio file is 1400 bytes. The length of an app-delivered audio file is fixed to 320 bytes.

- Use the TUYA_APP_Put_Frame interface to send the audio and video data that the camera collects to the buffer.

- NOTE: Currently, Tuya SDK only supports H.264 encoded video data.

Only the 16:9 video resolutions, namely, 720 p and 1080 p, are supported.

- In the TUYA_APP_Enable_Speaker_CB function, enable the speaker switch control.

- In the TUYA_APP_Rev_Audio_CB function, enable audio data play.

4.2 Local Video Development

The SDK library file has integrated the local storage function. You only need to send the audio and video data to ringbuffer and configure local storage. This section describes how to enable the local video storage function and configure it.

Development Process

- After Message Queuing Telemetry Transport (MQTT) is available, call the IPC_APP_Sync_Utc_Time function to synchronize the time with the server time. Use the callback function __IPC_APP_Get_Net_Status_cb to check whether MQTT is online, as shown in the following figure.

- NOTE: Before enabling the local video storage function, synchronize the time to prevent playback video locating failures due to incorrect time.

Time synchronization may fail. Call the function multiple times until time synchronization is successful.

- NOTE: The time zone is changed and delivered as follows:

If you change the time zone of your mobile phone before using the mobile phone to connect the device to the network, the time zone takes effect for the device. If you change the time zone of your mobile phone after the device has been connected to the network, the modification does not take effect for the device.

The time zone is subject to the time zone setting of your mobile phone and is independent of the app account region and time zone.

- Call the TUYA_APP_Init_Stream_Storage function to initialize the local storage function. By default, continuous storage is enabled. To enable event storage, add tuya_ipc_ss_set_write_mode (SS_WRITE_MODE_ALL) and tuya_ipc_ss_ set_write_mode (SS_WRITE_MODE_EVENT), as shown in the following figure.

- NOTE: After continuous storage is enabled, the SDK will automatically save videos to the SD card.

After the device starts, this function is called only once to initialize local storage.

Local storage only supports the SD card in FAT32 format. If the format of the device's SD card is incorrect, the error information will be reported to the app based on the DP setting for the SD card. When you tap the SD card setting in the app, the app displays a pop-up to inform you of the error. In this case, format the SD card.

- If event storage is enabled, the device calls the tuya_ipc_ss_start_event function to record a video, and the tuya_ipc_ss_stop_event function to stop recording. The maximum time of a single event recording is 10 minutes. If the tuya_ipc_ss_stop_event function is not called within 10 minutes after the tuya_ipc_ss_start_event function is called, the SDK will automatically stop video recording.

- Configure SD card check in the tuya_ipc_sd_get_status function as required.

- NOTE: If an incorrect SD card format, for example, NTFS, is detected, abnormal will be returned.

5 DP Development

5.1 Overview

Data points (DPs) are used to control various device functions. Based on the DPs and DP values delivered by the server, configure the device DPs and send the settings to the server. The tuya_ipc_utils.h file specifies general DPs to help you use the existing demo code to develop and configure DPs. The code of a customized DP is similar to that of a DP of the same data type.

DP Data Saving and Reading

- Set the __tuya_app_write_INT and __tuya_app_write_STR functions for DP data saving.

- Set the __tuya_app_read_INT and __tuya_app_read_STR functions for DP data reading.

5.2 OSD Watermark DP Development

- OSD watermark DP: 104

- The server sends a DP value to determine whether there is a time watermark in a video, and the device controls the watermark position and content.

5.3 PIR DP Development

- PIR DP: 152

- The server sends a DP value to enable or disable the device's PIR function and control the PIR sensitivity.

When the PIR function is triggered, the tuya_ipc_notify_door_bell_press function is called to report the collected camera image to the server.

- NOTE: It is recommended that the passive infrared (PIR) detection results be reported at 30s intervals.

5.4 Motion Detection Development

The SDK provides functions for motion detection switch control, sensitivity control, timing control, intermittent control, and data reporting. The device determines whether the alarm thresholds are exceeded.

5.4.1 Motion Detection Development Process

- Motion detection switch DP: 134
Motion detection sensitivity DP: 106

- When the camera motion detection function is enabled and the video image changes, the tuya_ipc_notify_motion_detect function is called to report the image.

5.4.2 Scheduled Motion Detection Development Process

- Define the following two DPs. The code is similar to that of the DPs of the same data type.

#define TUYA_DP_MOTION_TIMER_SET           "114"       /* alarm timer, STR, start time, end time */

#define TUYA_DP_ MOTION _ TIMER _SWITCH           "135"        /* motion detection mode selection, STR, true: scheduled motion detection, false: all-day motion detection */

- Function process

- After receiving the DP values, the device enables the scheduled motion detection function during the specified detection time.

- The device returns DP 135 true to inform the server that it has enabled scheduled motion detection.

- At the start time, for example, at 14:53, the device reports detected motion.

- At the end time, for example, at 19:00, the device returns DP 134 false to inform the server that it has disabled motion detection.

- The following are examples of data delivered from the server.

- "dps": {"114": "14:53 | 19:00"}
114 indicates motion detection time settings. 14:53 is the start time, and 19:00 is the end time.

- "dps": {"135": true}
135 indicates motion detection mode selection, and the value is true, indicating scheduled motion detection. The value false indicates all-day motion detection.

5.4.3 Intermittent Motion Detection

- Intermittent motion detection DP: 133

- The server sends DP 133 to enable intermittent motion detection and specify the detection intervals. For example, if the camera receives DP 133 with the value 5, it enables motion detection at 5 minute intervals, and detects motion for 5 minutes each time.

6 Doorbell DP Development

This chapter describes how to develop DPs for doorbell products, including low power-consuming sleep and doorbell wakeup. Skip this chapter if you are not developing doorbell product.

6.1 Low Power-consuming Sleep DP Development

Doorbell Sleep

- Add the following DP code.

- NOTE: DP 149 indicates the doorbell status. Its value can be 0 or 1. The value 0 indicates that the doorbell has entered the sleep mode, and the value 1 indicates that the doorbell has awakened.

The DP code is similar to that of the DP of the same data type.

#define TUYA_DP_DOOR_SLEEP                 "149"         /* Sleep mode, Boolean, true: not sleep, false: sleep */

- When entering the low power-consuming mode, the doorbell:

- Calls the tuya_ipc_book_wakeup_topic function to inform the server that it is about to enter the sleep mode.

- Calls the tuya_ipc_get_mqtt_socket_fd function to obtain the corresponding socket fd information.

- Calls the tuya_ipc_get_wakeup_data function to obtain wakeup data from the app. After the doorbell's low power-consuming chip connects to the server, the doorbell checks whether wakeup data has been received.

- Calls the tuya_ipc_get_heartbeat_data function to receive keeping alive packets from the server when it is in sleep mode. It is recommended that keeping alive packets be sent at 30s to 120s intervals.

- Sends DP 149 to inform the server that it has entered the sleep mode.

- Enters the sleep mode. Only the low power-consuming chip works to exchange heartbeat packets with the server.

Doorbell Wakeup

- When the doorbell receives wakeup data, it wakes up and returns DP 149 with the value 1.

- If you manually press the doorbell to wake it up, the doorbell calls the IPC_APP_trigger_door_bell and tuya_ipc_notify_door_bell_press functions to return DP 149 (low power-consuming, wakeup mode) and DP 136 (doorbell, ring), and push collected image information.

- Functions for querying in-app operations

- Callback function for obtaining audio and video on and off information: __TUYA_APP_p2p_event_cb

- Function for querying the number of device connections: tuya_ipc_get_client_conn_info

7 OTA DP Development

This chapter describes OTA, one of the most important device upgrade and maintenance methods. When the firmware needs to be upgraded, the SDK uses callback functions to inform the device to perform an OTA upgrade.

7.1 OTA Development Process

- The OTA request callback function is IPC_APP_Upgrade_Inform_cb (), including the firmware download URL, MD5 for file verification, and file size.

- NOTE: The device calculates the MD value of the downloaded firmware and compares the calculated value with the MD5 value sent from the server.

If the values are the same, the firmware is correct.

- Releasing or optimizing system memory resources: Call the tuya_ipc_ss_set_write_mode function to disable local recording, and call the tuya_ipc_ss_uninit() and tuya_ipc_tranfser_quit functions to deinitialize local storage and P2P function of the SDK to release as many memory resources as possible.

- NOTE: Optimize the memory resources before calling the tuya_ipc_upgrade_sdk function.

- Call back the __IPC_APP_get_file_data_cb function to download the firmware.

- NOTE: The function contains input and output parameters for writing and reading data, respectively. If data has been written successful, the output parameter returns 0.

- Call back the tuya_ipc_upgrade_sdk function.

- NOTE: The first parameter notifies the SDK and app of the overall OTA upgrade progress. After the firmware is downloaded, the second parameter shows the download result.

- Specify the new firmware path in the __IPC_APP_upgrade_notify_cb function.

- NOTE: After the firmware is downloaded, specify the firmware path in the function and reboot the system.

- If the OTA upgrade fails, add a reboot operation in the __IPC_APP_upgrade_notify_cb function.

- NOTE: Deinitialization is irreversible. If the OTA upgrade fails, the device must be restarted.

- During the firmware upgrade, the progress bar between 92% and 98% in the app indicates that the firmware has been downloaded. The app is waiting for the device to restart and report the new firmware version number. After the app receives the new firmware version number, it will display an upgrade success.

8 Cloud Storage Capability Development

The SDK library file has integrated the cloud storage capability. After you purchase the cloud storage service, the SDK will send your audio and video information in ringbuffer to the cloud for storage. The storage time depends on the cloud storage service you have purchased.

8.1 Development Process

- Provide the device's PID and contact a project manager to open the cloud storage service. The cloud storage price during the development phase is only valid for the testing period, and the original service price will be restored after the project development is completed.

- After purchasing the service, call the cloud storage initialization function TUYA_APP_Enable_CloudStorage.

- By default, the SDK uses software encryption. The encryption interface is OpensslAES_CBC128_encrypt. The cipher block chaining (CBC) mode of the advanced encryption standard (AES) is used.

If the software encryption causes a high load to the MCU, change to hardware encryption. Use the SoC hardware encryption channel and the PKCS #5 or PKCS #7 method.

- Full-time video cloud storage: By default, the ringbuffer data sent by the camera is uploaded to the cloud. In addition, when the camera detects changes in the video image, it calls the tuya_ipc_cloud_storage_event_start and tuya_ipc_cloud_storage_event_stop functions to start and stop cloud storage video tagging, respectively. A tag marks a video segment with changes, and the tagged video segment will be added to a list.

- NOTE: If the device does not call the tuya_ipc_cloud_storage_event_stop function within 5 minutes after it calls the tuya_ipc_cloud_storage_event_start function, the SDK will automatically stop the cloud storage video tagging.

- Event video cloud storage: When the camera detects changes in the video image, it calls the tuya_ipc_cloud_storage_event_start function. The SDK uploads the audio and video information to the cloud. When the video image is static, the camera calls the tuya_ipc_cloud_storage_event_stop to stop uploading the video to the cloud.

The uploaded video will be added to a list.

- NOTE: If the device does not call the tuya_ipc_cloud_storage_event_stop function within 5 minutes after it calls the tuya_ipc_cloud_storage_event_start function, the SDK will automatically stop video uploading to the cloud.

9 Echo Show and Chromecast DP Development

The Echo Show and Chromecast DPs are used to display audio and video data collected by the camera on Amazon's or Google's smart devices. This chapter describes how to develop the DPs. Before development, build a test environment.

9.1 Echo Show DP Development Process

- Provide the device's PID and contact a project manager to enable the Echo Show access service.

- Call the Echo Show initialization function TUYA_APP_Enable_EchoShow_Chromecast.

- The coding format of Echo Show audio files must be PCM or G711U.

- You are advised to set the video channel to the substream: es_param.vchannel = E_CHANNEL_VIDEO_SUB.

- Echo Show display optimization suggestions:

- When the camera is working, it sends audio and video streams continuously to ringbuffer.

- Control the substream rate at 300 kbit/s to 512 kbit/s.

- If the Echo Show speaker plays the video slowly, check whether the audio format is PCM or G711U.

- Audio and video streaming on the Echo Show speaker automatically stops after 5 minutes.

9.2 Chromecast DP Development Process

- Provide the device's PID and contact a project manager to enable the Chromecast access service.

- Call the Chromecast initialization function TUYA_APP_Enable_EchoShow_Chromecast.

- Set the additional audio channel E_CHANNEL_AUDIO_2RD and apply for memory for the channel.

- Chromecast audio requirements 
a. Audio file format: AAC (ADTS)
b. Sampling rate: 8 kHz
c. Bit width: 16 bits
d. Monophonic audio
e. PTS unit: microsecond (μs). If the decoded time unit is milliseconds, multiply the value by 1,000 to output a result in microseconds.

- If the device cannot provide the AAC audio data, download the FDK AAC open-source library and convert the PCM audio data to AAC (ADTS) data. The library file is about 4 MB to 5 MB. Contact Tuya FAE for the audio conversion sample code.

- If the following device log is displayed, the audio signal is incorrect and the server will disconnect from the device.

- Chromecast is sensitive to audio and video time. The audio PTS and the video PTS must be consistent and use microseconds as the unit. If the video time format is UTC+microseconds, the audio time format must be the same.

- Suggestions on improving Chromecast video fluency:

- Increase the frame rate and the I frame frequency.

- Chromecast only supports a 7s wait time. Ensure that the device has a fast response capacity to meet Chromecast's requirement.

- By default, the SDK uses the substream video channel. Reduce code rate to the range of 300 kbits/s to 500 kbits/s.

- Verify the Chromecast DP. When the device receives a command from the server and replies, run the ffplay -v debug URL in the log command to test Chromecast DP on your computer. In the following example, the URL in the log is rtmp://alxs1.tuyaus.com:1935/hls/8316047019281803308. If the Chromecast DP is normal, you can get the audio and video information from the device.

10 Head Shake Camera Movement Development

Head shake camera functions include button-controlled movement, button-controlled stop, adding a preset, deleting, and calling, and patrol configuration.

10.1 Head Shake Camera DP Description

- Provide the device's PID and contact a project manager to enable head shake camera functions.

- Movement related functions, including button control, in-position, and self-check, are user-defined.

| Head Shake Camera DP | Description |
| --- | --- |
| 119 | PTZ movement control. Versions earlier than V4.0: 0: up; 1: up right; 2: right; 3: down right; 4: down; 5: down left; 6: left; 7: up left V4.0 or later: 0: up right; 1: right; 2: down right; 3: down; 4: down left; 5: left; 6: up left; 7: up |
| 116 | PTZ rotation stop, Boolean |
| 132 | PTZ self-check. The value can be: 0: disable 1: enable The default value is 0. |
| 161 | Movement tracking, Boolean. The value can be: true: enable false: disable |
| 174 | Patrol switch. The value can be: 0: disable 1: enable |
| 175 | Patrol type mode. The value can be: 0: panoramic patrol 1: preset patrol |
| 176 | Patrol time mode. The value can be: 0: all-day patrol 1: scheduled patrol |
| 177 | Patrol time setting. |
| 178 | Preset operation, string. The value type:1 indicates adding, and type:2 indicates deleting. Different data types have different strings. |
| 179 | Patrol status query command. The returned value can be: 0: panoramic patrol 1: preset patrol 2: non-patrol mode |

10.2 Interface Description

| Interface | Description | Remarks |
| --- | --- | --- |
| IPC_APP_ptz_start_move | Movement control source code | The input parameter is direction. For details, see section 10.1 "Head Shake Camera DP Description.” |
| IPC_APP_ptz_stop_move | Motor stop source code |  |
| IPC_APP_ptz_check | Motor self-check code |  |
| IPC_APP_track_enable | Tracking interface | 1: enable 0: disable |
| IPC_APP_set_patrol_mode | Patrol mode configuration interface. Design the bottom-layer patrol code based on your requirements. | Currently, two patrol modes are supported. |
| IPC_APP_set_patrol_switch | Patrol switch | The patrol mode and time settings are available only when this switch is enabled. |
| IPC_APP_ptz_preset_reset | Interface for synchronizing the preset information on the server to the local configuration profile of the device | The tuya_ipc_preset_get function typically calls the interface to synchronize the server-side information during device startup. |
| IPC_APP_set_patrol_tmode | Interface for patrol time mode configuration | 0: all-day patrol 1: timed patrol, which can be configured across days. If the start time and the end time are the same, it is equivalent to the all-day patrol mode. |
| IPC_APP_set_patrol_time | Patrol timer setting interface |  |
| IPC_APP_set_preset | Preset setting interface | 1: add 2: delete 3: call |

10.3 DP Configuration Protocol Examples

Button control configuration

{
    "data":{
        "dps":{
            "119":"6" //DP 119. The SDK will convert 6 to 1 and then convert 1 to 6 when required. For details, see the demo.
        }
    },
    "protocol":5,
    "t":1553674406
}

Stopping button control

{
    "dps":{
        "116":true
    }
}

Motor self-check

{
    "data":{
        "dps":{
            "132":true
        }
    },
    "protocol":5,
    "t":1553675285
}

Tracking enabling

{
    "data":{
        "dps":{
            "161":true
        }
    },
    "protocol":5,
    "t":1553675430
}

Patrol mode JSON format configuration

{
    "data":{
        "dps":{
            "175":"0"  // DP 175. 0: panoramic patrol; 1: preset patrol
        }
    },
    "protocol":5,
    "t":1553673946
}

Patrol switch JSON configuration

{
    "data":{
        "dps":{
            "174":true
        }
    },
    "protocol":5,
    "t":1553675538
}

Patrol time mode selection

{
    "data":{
        "dps":{
            "176":"0"
        }
    },
    "protocol":5,
    "t":1553675681
}

Patrol time setting

{
    "data":{
        "dps":{
            "177":"{"t_end":"18:02","t_start":"16:02"}"
        }
    },
    "protocol":5,
    "t":1553675877
}

Adding a preset (app to device)

{
    "data":{
        "dps":{
            "178":"{"data":{"name":"收藏点2"},"type":1}"  //Add a preset.
        }
    },
    "protocol":5,
    "t":1553676980
}

Adding a preset (device to server)

{
    "v":1,
    "devId":"6c724ed0773b82fc50atqh",
    "mpId":2,
    "pic":"preset_1553676982.jpg",
    "name":"收藏点2",
    "pos":{
        "pan_pos":3051,
        "tilt_pos":114
    }
}

Calling a preset

{
    "data":{
        "dps":{
            "178":"{"data":{"mpId":"1"},"type":3}" // Call the first preset.
        }
    },
    "protocol":5,
    "t":1553676879
}

Deleting a preset (app to device)

{
    "data":{
        "dps":{
            "178":"{"data":{"num":1,"sets":[{"devId":"0000016vns","mpId":"1"}]},"type":2}"
        }
    },
    "protocol":5,
    "t":1553677541
}

Deleting a preset (device to server)

{
    "v":1,
    "id":"0000016vns",
    "t":1553677542
}